Application Review Committee

Division Comments- [BUILDING DIVISION]

Development Services Department

300 N. Coast Hwy, Oceanside, CA 92054   |  (760) 435-4373

To be filled out by Project Planner

Project/property address and/or APN: Village Commercial Planning Area of the El Corazon Specific Plan (portion of APN:162-082-51)

Project description: Mixed-Use development consisting of 268 residential apartments integrated with 4,756 s.f. of ground-level commercial space within the El Corazon Specific Plan Area

.  [Full project description is in TrakIt]

To be filled out by ARC Division Reviewer

☐  1st    ☒  2nd    ☐  3rd    ☐  4th   Review

Staff member:

Phone number:

E-mail:

☐  Review Comments       ☒  Conditions of Approval

(include attachments/forms on TRAKiT, if applicable):

[insert/paste comments in Times New Roman, 13 pt font, numbered, 1.5 spacing]

1)          The granting of approval under this action shall in no way relieve the applicant/project from compliance with all Current State and local building codes.

The 2016 triennial edition of the California Code of Regulations, Title 24 (California Building Standards Code) applies to all occupancies that applied for a building permit on or after January 1, 2017, and remains in effect until the effective date of the 2019 triennial edition which will be January 1, 2020.

Beginning on January 1, 2020, Oceanside Development Services (ODS) is required by State law to enforce the 2019 Edition of California Building Standards Codes (a.k.a., Title 24 of the California Codes of Regulations).

Every three years, the State adopts new model codes (known collectively as the California Building Standards Code) to establish uniform standards for the construction and maintenance of buildings, electrical systems, plumbing systems, mechanical systems, and fire and life safety systems. Sections 17922, 17958 and 18941.5 of the California Health and Safety Code require that the latest edition of the California Building Standards code and Uniform Housing Code apply to local construction 180 days after publication.

There are 12 parts to Title 24 and the applicable parts for most Building Division permit applications are listed below.

- Part 2: The 2019 California Building Code (CBC).

- Part 2.5: The 2019 California Residential Code (CRC).

- Part 3: The 2019 California Electrical Code (CEC).

- Part 4: The 2019 California Mechanical Code (CMC).

- Part 5: The 2019 California Plumbing Code (CPC).

- Part 6: The 2019 California Energy Code

- Part 9: The 2019 California Fire Code (CFC)

- Part 11: The 2019 California Green Building Standards Code (CALGreen Code) This Part is known as the California Green Building Standards Code, and it is intended that it shall also be known as the CALGreen Code.

All architects, engineers, designers, developers, owners and contractors MUST be familiar with the codes in effect at the time of plan submittal. ODS, as required by State law, CANNOT approve projects that do not comply with the codes in effect at the time of plan submittal. All projects submitted before or on December 31, 2019 are permitted to comply with the 2016 Edition of the California Building Standards Code.

- The granting of approval under this action shall in no way relieve the applicant/project from compliance with all Current State and local building codes.

3)      The building plans for this project are required by State law to be prepared by a licensed architect or engineer.

4)      Compliance with the Federal Clean Water Act (BMP’s) shall be demonstrated on the plans.

5)     All outdoor lighting shall meet Chapter 39 of the City Code (Light Pollution Ordinance) and shall be shielded appropriately.

6)      Separate/unique addresses may be required to facilitate utility releases.  Verification that the addresses have been properly assigned by the City’s Planning Division shall accompany the Building Permit application.

7)      A form or foundation survey shall be required prior to the placement of concrete to show the location of the new structure in respect to the property lines, known easements, and known setback lines. By obtaining a form survey the location of the foundation is checked prior to the placement of concrete, and can save costly corrective measures in case of an encroachment of a property line

8)      The 2016 California Energy Code requires rooftop solar zones.   The requirements for solar ready buildings are mandatory measures for newly constructed single-family residences and low-rise multifamily residential buildings.  Plans must be submitted with the minimum required solar zones indicated on each required structure.  2016 California Energy Code, Section 110.10 A-D. Section 110.10 Mandatory Requirements for Solar Ready Buildings

(a) Covered occupancies.

- Single-family residences. Single-family residences located in subdivisions with ten or more single-family residences and where the application for a tentative subdivision map for the residences has been deemed complete by the enforcement agency shall comply with the requirements of Sections 110.10(b) through 110.10(e)

- Low-rise multifamily buildings. Low-rise multifamily buildings shall comply with the requirements of Sections 110.10(b) through 110.10(d).

- Hotel/motel occupancies and high-rise multifamily buildings. Hotel/motel occupancies and high-rise multifamily buildings with ten habitable stories or fewer shall comply with the requirements of Sections 110.10(b) through 110.10(d).

- All other nonresidential buildings. All other nonresidential buildings with three habitable stories or fewer shall comply with the requirements of Sections 110.10(b) through 110.10(d).

9)    Exterior walls of all buildings shall comply with CBC table 705.2

10)   CBC 1027.5 Location.  Exterior exit stairways and ramps shall have a minimum fire separation distance of 10 feet (3048 mm) measured at right angles from the exterior edge of the stairway or ramps, including landings, to:

1. Adjacent lot lines.

2. Other portions of the building.

3. Other buildings on the same lot unless the adjacent building exterior walls and openings are protected in accordance with Section 705 based on fire separation distance. For the purposes of this section, other portions of the building shall be treated as separate buildings.

11)     1023.7 Interior Exit Stairway and Ramp Exterior Walls
Exterior walls of the interior exit stairway or ramp shall comply with the requirements of Section 705 for exterior walls. Where nonrated walls or unprotected openings enclose the exterior of the stairway or ramps and the walls or openings are exposed by other parts of the building at an angle of less than 180 degrees (3.14 rad), the building exterior walls within 10 feet (3048 mm) horizontally of a nonrated wall or unprotected opening shall have a fire-resistance rating of not less than 1 hour. Openings within such exterior walls shall be protected by opening protectives having a fire protection rating of not less than 3/4 hour. This construction shall extend vertically from the ground to a point 10 feet (3048 mm) above the topmost landing of the stairway or ramp, or to the roof line, whichever is lower.

12)      Site development, parking, access into buildings and building interiors shall comply WITH ALL CURRENT State of California Accessibility Code where required.

- Buildings or portions of buildings and facilities within the scope of this chapter shall be accessible to persons with disabilities. Each building on a building site shall be considered separately when determining the requirements contained in this chapter, except when calculating the number of units which must comply with Section 1102A.3.1. Dwelling units within a single structure separated by firewalls do not constitute separate buildings.

- Newly-constructed covered multifamily dwellings as defined in this chapter, include, but are not limited to, the following:

- Apartment buildings with 3 or more dwelling units including timeshare apartments not considered a place of public accommodation or transient lodging as defined in Health and Safety Code Section 19955 (a), and Chapter 2 of the California Building Code.

- Condominiums with 4 or more dwelling units including timeshare condominiums not considered a place of public accommodation or transient lodging as defined in Health and Safety Code Section 19955 (a), and Chapter 2 of the California Building Code.

- Lodging houses, as defined in Chapter 2 of the California Building Code, used as a residence with more than 3 but not more than 5 guest rooms.

- Congregate residences, as defined in Chapter 2 of the California Building Code, with 3 or more sleeping units.

- Dwellings with 3 or more efficiency units, as defined in Chapter 2 of this code, or Section 17958.1 of the California Health and Safety Code.

- Shelters for homeless persons, not otherwise subject to the disabled access provisions of the Division of the State Architect-Access Compliance (DSA-AC).

- Dormitories, as defined in Chapter 2 of this code, with 3 or more guest rooms as defined in Chapter 2 of the California Building Code.

- Timeshare dwellings with 3 or more units, not considered a place of public accommodations or transient lodging as defined in Health and Safety Code Section 19955 (a), and Chapter 2 of the California Building Code.

- Other Group R occupancies in covered multifamily dwellings which are regulated by the Office of the State Fire Marshal. See Section 1.11.

- Public housing as defined in Chapter 2 of this code is subject to provisions of the Division of the State Architect (DSA-AC) in Chapter 11B. Newly constructed covered multifamily dwellings, which can also be defined as public housing, shall be subject to the requirements of Chapter 11A and Chapter 11B.

- 1102A.3.2 Multistory dwelling units in buildings with one or more elevators

Multistory dwelling units contained in buildings with elevators shall comply with this section. For multistory dwelling units in buildings with elevators, the story of the unit that is served by the building elevator is considered a ground floor and the primary entry floor to the unit and shall comply with the following:

- At least 1 powder room or bathroom shall be located on the primary entry level.

- At least 1 kitchen shall be located on the primary entry level.

- All rooms or spaces located on the primary entry level shall be served by an accessible route and shall comply with Division IV.

### 1109A.3 Required accessible parking spaces

Accessible parking spaces shall be provided at a minimum rate of 2 percent of the covered multifamily dwelling units. At least one space of each type of parking facility shall be made accessible even if the total number exceeds 2 percent.

### 1109A.7 Location of accessible parking spaces

The location of accessible parking spaces shall comply with the following:

- Accessible parking spaces shall be located on the shortest possible accessible route to an accessible building, or covered multifamily dwelling unit entrance. All van accessible spaces may be grouped on one level of a multilevel parking facility.  Please illustrate compliance on the plans.

- When parking facilities are located adjacent to a building with multiple accessible entrances, accessible parking spaces shall be dispersed and located near the accessible building entrances.

- When practical, the accessible route shall not cross lanes for vehicular traffic. When crossing vehicle traffic lanes is necessary, the accessible route shall be designated and marked as a crosswalk.

- Parking facilities that do not serve a particular building shall have accessible parking spaces located on the shortest possible accessible route to an accessible pedestrian entrance of the parking facility.

- Accessible parking spaces shall be located so that persons with disabilities are not compelled to wheel or walk behind parked cars other than their own.

Exception: When the enforcement agency determines that compliance with this section or providing equivalent facilitation would create an unreasonable hardship, parking spaces may be provided which would require a person with physical disabilities to wheel or walk behind other than accessible parking spaces.

12)      A complete set of Soil Reports, Structural Calculations, Energy Calculations, & California Title 24 Energy Form(s) shall be required at time of plans submittal to the Building Division for plan check.

13)        City of Oceanside Enforces the 2016 California Green Building Standards Code.  A Construction Waste Management Plan shall be required at time of plans submittal to the Building Division for plan check.

5.408.1 Construction waste diversion.  Recycle and/or salvage  for reuse a minimum of 65 percent of the nonhazardous construction and demolition  waste  in  accordance  with  Section  5.408.1.1,  5.408.1.2  or 5.408.1.3; or meet a local construction and demolition waste management ordinance, whichever is more stringent.

5.408.1.1 Construction waste management plan. Where a local jurisdiction does not have a construction and demolition waste management ordinance that is more stringent, submit a construction waste management plan that:

1. Identifies the construction and demolition waste materials to be diverted from disposal by efficient usage, recycling, reuse on the project or salvage for future use or sale.

2. Indicates if construction and demolition waste materials will be sorted on-site (source-separated) or bulk mixed (single stream).

3. Identifies diversion facilities where construction and demolition waste material collected will be taken.

4. Specifies that the amount of construction waste and demolition materials diverted shall be calculated by weight or volume, but not by both.

14)      Plans must specify, as applicable, the type of automatic sprinkler system – NFPA 13, NFPA 13R, or NFPA 13D – installed in each building.

15)     For mixed-occupancy, multistory buildings: Calculation of total allowable building area and allowable building area per story per CBC 506.2.4 shall be provided.

16)     Electric vehicle (EV) charging for new construction. New construction shall comply with sec. 4.106.4.1 and 4.106.4.2 to facilitate future installation and use of EV chargers.  Electric vehicle supply equipment (EVSE) shall be installed in accordance with California Electrical Code (CEC), Article 625.

17)      Solar ready rooftop required. Low-rise and high-rise multifamily buildings, hotel/motel occupancies and nonresidential buildings. The solar zone shall be located on the roof or overhang of the building or on the roof or overhang of another structure located within 250 feet of the building or on covered parking installed with the building project and have a total area no less than 15 percent of the total roof area of the building excluding any skylight area. The requirements for solar ready buildings are all mandatory, so there are no prescriptive and performance compliance paths. Since the provisions are mandatory, there are also no tradeoffs allowed, and applicants must demonstrate compliance with each measure.

18)      4.410.1 Operation and maintenance manual.  At the time of final inspection, an operation and maintenance manual shall be provided to the building occupant or owner.

19)     4.406.1 Rodent proofing. Annular spaces around pipes, electric cables, conduits or other openings in plates at exterior walls shall be protected against the passage of rodents by closing such openings with cement mortar, concrete masonry or similar method acceptable to the enforcing agency.

20)     4.503.1 Any installed gas fireplace shall be a direct-vent sealed-combustion type. Any installed woodstove or pellet stove shall comply with U.S. EPA New Source

Performance Standards (NSPS) emission limits as applicable, and shall have a permanent label indicating they are certified to meet the emission limits. Woodstoves, pellet stoves and fireplaces shall also comply with applicable local ordinances.

21)    San Diego County Department of Environmental Health approval is required for all new food businesses and public pools.

22)    The construction documents and/or site plan should indicate the location and required number of designated parking stalls.  These parking spaces should be marked “CLEAN AIR/VANPOOL/EV.”    The markings should be visible when a clean air vehicle is parked.  In other words, if the front of the vehicle goes into the parking stall first, the markings should be visible at the back end of the vehicle. Lettering should be at least 8 inches high. The CLEAN AIR/VANPOOL/EV parking stalls may be located anywhere on the site and do not require a preferential location.  Refer to Table 5.106.5.2 in CAL Green to ensure that the correct number of designated parking stalls is provided. Include all parking spaces in the calculation. 2016 Cal Green Section 5.106.5.2

23)     All electrical, communication, CATV, etc. service lines within the exterior lines of the property shall be underground (City Code Sec. 6.30).

24)      The developer shall monitor, supervise and control all building construction and supportive activities so as to prevent these activities from causing a public nuisance, including, but not limited to, strict adherence to the following:

Sec. 6.25. - Construction hour limitations.

It shall be unlawful to operate equipment or perform any construction in the erection, demolition, alteration, or repair of any building or structure or the grading or excavation of land during the following hours:

(1) Before 7:00 a.m. and after 7:00 p.m. Monday through Saturday.

(2) All day on Sunday; and

(3) On any federal holiday.

(b) Exceptions.

(1) An owner/occupant or resident/tenant of residential property may engage in a home improvement project between the hours of 9:00 a.m. and 5:00 p.m. on Sundays and holidays provided the project is for the benefit of said residential property and is personally carried out by said owner/occupant or resident/tenant.

(2) The building official may authorize extended or alternate hours of construction for the following circumstances:

a. Emergency work.

b. Adverse weather conditions.

c. Compatibility with store business hours.

d. When the work is less objectionable at night than during daylight hours.

e. Per direction of the city manager's office for projects that have been determined that rapid completion is in the best interest of the general public.