March 22, 2021

Application Name: Tri City Medical Center Psychiatric Health Facility

Application Number: D21-00004

Project Description: Psychiatric Health Facility

Engineering Review (1st. submittal)

Comply with the following comments:

(Grading Plan)

- Provide Legal Description on title sheet of civil plan.

- Provide APN numbers and owners names of adjacent lots.

- Provide street section of Waring Road with dimensions and labeling of the public right of way.

- Provide name, address, and phone number of surveyor and applicant.

- Provide date survey was performed.

- Provide a basis of bearings for the shown survey.

- Provide Sight distance triangles per section 400 of the Caltrans Highway Design Manual and City of Oceanside Standard Drawing T-1 for the drive entrance at Waring Rd.

- Provide all meets and bounds information along the shown property lines.

- Provide curb radius information for the drive entrance at Waring Road.

- Provide curb radius for proposed curbs within drive aisles.

- Minimum drive aisle width is 28’, please confirm current design with fire department.

- Provide a preliminary estimate of earthwork quantities.

- Provide gross/net acreage/square footage of lot(s)

- Provide finished pad elevation for proposed structure.

- Provide finished grade elevations alongside the top of wall elevations for all proposed walls.

- Clearly identify the type of wall to be constructed for all proposed walls.

- Provide site specific FEMA information including map and panel no.

- Identify all existing and proposed easements.

- Provide ADA accessible pedestrian ramps at the drive entrance to Waring Road.  Refer to San Diego County Region Standards.

- Clearly identify all proposed ADA accessible pedestrian ramps on site with the applicable County Regional Standard.

- Provide a San Diego County Regional Standard for the proposed drive way.

- Clearly identify the location of the ADA path of travel within the drive area in the northerly portion of the parking area.  Provide rates of grade to show ADA compliancy.

- Refer to San Diego County Regional Standards in lieu of San Diego County Standards.

(SWQMP)

- Submit a SWQMP for review.

(Drainage Study)

- Submit a drainage study for review.

If you have any questions regarding this review please do not hesitate to contact me.

Michael Strizic

760-435-3564

mstrizic@oceansideca.org