November 9, 2021

Application Name: Tri City Medical Center Psychiatric Health Facility

Application Number: D21-00004

Project Description: Psychiatric Health Facility

Engineering Review (2nd. submittal)

Comply with the following comments:

(Grading Plan)

- Provide Legal Description on title sheet of civil plan.

- Provide APN numbers and owners names of adjacent lots.

- Provide street section of Waring Road with dimensions and labeling of the public right of way.

- Provide name, address, and phone number of surveyor and applicant on the civil sheets of the plan set.

- Provide date survey was performed on the civil sheets of the plan set.

- Provide a basis of bearings for the shown survey on the civil sheets of the plan set.  The information provided in the response letter is benchmark information and not the requested basis of bearings.  Please consult with your surveyor for more details.

- Provide Sight distance triangles per section 400 of the Caltrans Highway Design Manual and City of Oceanside Standard Drawing T-1 for the drive entrance at Waring Rd.

- Provide all metes and bounds information along the shown property lines.

- Provide curb radius information for the drive entrance at Waring Road.

- Provide curb radius for proposed curbs within drive aisles.  Curb radii should be labeled on the civil sheets of the plan set.  Provide a line and curve table if necessary.

- Provide a preliminary estimate of earthwork quantities as an independent note on the title sheet on the civil plan set.

- Provide gross/net acreage/square footage of lot(s).  This should be placed as an independent note on the title sheet of the civil plan set.

- Provide finished grade, top of wall, and bottom of wall elevations for al proposed walls.

- Provide site specific FEMA information including map and panel no. as an individual note on the title sheet of the civil plan set.

- Confirm that the rates of grade and geometrics of the existing pedestrian ramps at the drive entrance to Waring Road meet the current ADA requirements.  If they do not meet the current ADA requirements they will need to be reconstructed in accordance with the San Diego County Region Standards.

- Provide a San Diego County Regional Standard for the proposed drive way.  A City of Oceanside Standard Driveway may be implemented, however ADA pedestrian ramps are still required.

- Clearly identify the location of the ADA path of travel within the drive area in the northerly portion of the parking area where they are located along the eastern façade of the building.  Provide rates of grade to show ADA compliancy.

(SWQMP)

- Submit a SWQMP for review.  The City of Oceanside is the governing agency for the submittal and review of the SWQMP.

(Drainage Study)

- Submit a drainage study for review.

If you have any questions regarding this review please do not hesitate to contact me.

Michael Strizic

760-435-3564

mstrizic@oceansideca.org