ALTA CONSULTANTS

PLANNING    ENGINEERING    SURVEYING

1283 E. Main Street, Suite 109, El Cajon, CA. 92021 Tel. (858) 581-6101 Fax. (619) 749-8806

State License R.C.E. 38606 and L.S. 5335

November 3, 2021                                                                                                            Job No. 499-01

APN: 165-493-47-00

Letter of Description & Justification for 3710 Vista Way

Applications for TM21-00002 And D21-00005

This project is a common interest development for 12 detached one-story units of senior housing & (2) two Triplexes for a total of 18 units with (1) one attached garage for each unit, (1) one assigned parking space for each unit, for (2) two parking spaces per unit. In addition, this project proposes 7 additional guest parking spaces (2) of which are van accessible ADA spaces, (2) are compact spaces and (3) are standard parking spaces. The total parking spaces proposed for this site is 43 for a parking density 2.43 spaces per unit. No off-site parking is included in this parking count. This development requires 300 SF of open space per unit of which 50% must be common usable open space. The common usable open space required for this project is 2,700 SF. This project proposes 2,724 SF of common usable open space. CCR’s will be prepared for this project prior to occupancy. This project includes affordable housing. The project address is 3710 Vista Way adjacent to 78 Freeway. The project is located along the north side of Vista Way, at the northwest corner of  Vista Way and North Way. The 78 freeway runs parrallel with Vista Way’s southerly right-of-way. Access to the site is from Vista Way. The project is on 1.97-acre lot with no existing visible improvements other than some retaining walls. This site was previously part of a residential development with homes and streets. These improvements have been removed from the site. There is an existing manufactured slope along the north and west side of the project with several large existing trees. The remainder of the site is relatively flat. The site slopes west to east towards the southeast corner of the site (the northwest corner of Vista Way and North Way) where there is an existing cross-gutter that conveys runoff easterly across North Way. Both Vista Way and North Way are fully improved streets no new street dedications or improvements are proposed by this project.

This project will comply with the current State of California MS-4 permit. This project has minmualist areas adjacent to the street right-of-ways that will sheet flow into the existing curb and gutter. The roof gutters will discarge into landscaped areas then sheet flow into a paver private street and driveway that will collect the storm runoff into a gravel layer under pavers for flow control hydro-modification. The storm runoff will be routed to modlar wetland for pollution control. There will be storm water flow and pollution control compliance for this project.

The only improvements proposed within the public right-of-way are utility connections, Private Street “A” cross-gutter and curb returns and private storm drain connections to the existing public storm drain facilities. No offsite drainage flows onto the site, except at the west side of the project where a small amount of slope within the public right of way flows onto the site. There is one existing water service located on Vista Way this project proposes that this existing water service be reserved for irrigation purposes. This project proposes (2) new water services on Vista Way. Each service to supply water to half the proposed units (9 units each). The units will be individually metered by on-site private water meters. There are (2) two proposed private sewer mains which connect to the existing sewer in Vista Way with proposed manholes. Each service to supply sewer to half the proposed units (9 units each). Electrical, phone, cable and gas connections are proposed to be undergrouded and site improvements will be designed by others at a later date.

Proposed site acces is from Private Street “A” from Vista Way. This driveway design directly provides access to the 12 single famile units and has a 40 foot radius cul-de-sac at its east end to provide an emergency vehicle turn-a-round. At the east end of the cul-de-sac proposed Private driveway “B” provides access to the two triplexs (units 13-18) atrash enclosure and a parking lot.

This development requires several deviations from standards including the following:

- 2-car garages: Article 31 of the zoning Ordinance requires that all single-family homes have 2-car garages. This site is a common interest development. Although 12 of the units are proposed as detached units, they a part of a larger condominium type development. This aite has a covenant that restricts development to single story construction. Also, this site with its shape and terrain is restricted in the type of development that is possible. The single-family units are proposed to be separated by 11’ minimum. One detached parking space will be located within the space between units. These detached spaces will be assigned to the adjacent units. All units will have (1) one garage parking space and (1) one detached parking space for a total of (2) two parking spaces.

- Retaining Walls: the zoning Ordinance restricts the height of walls to 6 feet. A C-4 retaining wall maximum height of 5.33 feet is proposed along the street right of way of Vista Way and North Way. An up to 15-foot heigh retaining wall is proposed behind the proposed units on the west and north side of the site. These walls are needed to provide rear yard space.

- Setbacks: 1050(AA), 1050(DD) and 1050(L). The setbacks required for this site are 20’ from Vista Way, 15’ rear and 15’ from North Way; the garages from the private street  setback is 20’. Several of the units encroach into the setbacks. The reason for this the shape of the lot. The site is left over from the construction of Freeway 78 and Vista Way. The lot is long and shallow stretching along Vista Way. Access to Vista Way is limited due to the traffic and the separation needed from North Way.

- Private Road: Engineering Design manual Section 3.4, B1 requires that the the minimum centerline radius be 100’ and the minimum vertical curve length be 100’. Do to the previous described site restrictions this development proposes a centerline radius of 60’ for Private Street “a” and a minimum vertical curve length of 50’.

- Rear Yard: 1050(T) requires a minimum year yard of 15’. Do to previously described site restrictions this development is requesting that this requirement be waived.

This site’s design provides several benefits to the community. This is an infill project that removes a vacant site for the community eliminating the adverse possibilities that can happen on vacant sites such as trash and vagrancy. Detached one-story independent single-family senior housing. Affordable senior housing that is not in a multi-unit building. The proposed units will be sized at approximately 1,100 SF providing an easily maintained compact unit with individual rear yards. In addition once these units are occupied there will be an increase in the City of Oceanside’s tax base along with the positive impact for local Oceanside businesses with the residences that this project will provide

For the developer the benefits include the economic return for selling the units once the project is completed and the knowledge that this development greatly benefits the Oceanside community.